Main dimensions

Length over all approx.				69,38 m

Length car deck					63,80 m

Beam extreme					15,50 m

Depth moulded to main deck				5,10 m

Design draught, mld max				2,80 m

Max loaded draught, mld max 				3,00 m

Capacities Lane capacity min 				342 m

Passenger capacity summer 				540 pers.

Crew						10 pers.

Class: DNV ✠ 1A1 – CAR FERRY A – E0 – BATTERY SAFETY – BIS

Speed and Power

Power output , diesel generators			3700 kW

Power output , battery 	 			3300 kW

Total long term output, approx.				4 000 kW

Generators & batteries

Trial speed on design draught min			15,0 knots

Thrusters

Forward, 2x FP electric driven bow thruster 600 kW

Navigation & communication

Radars, DGPS, CCTV, auto pilots, gyro and chart system installed as an integrated system. All communication equipment to be in accordance with the requirements described in SOLAS Ch.4 for sea area A2. The wheelhouse to fulfil XSYS-NEQ 1 and XAUT-UMS notation for one man operation both at sea and during manoeuvring.

Propellers

2x azipull thrusters 1700 kW at 1600 RPM.

Diesel Generators

For electric power production, 3x diesel generator sets, each with a

capacity of about 1350- 1450 kVA (1200-1300 kW) at approx.

1500/1800 RPM

Emergency Diesel Generators

1x emergency diesel generator set, with a capacity of about 165 kVA at 1800 RPM

Propulsion support batteries

- Total capacity: 3300 kW

General data about the design & design highlights

Our PK-259.1 (MF Herjólfur) is a tailor-made design for the Vestmannaeyjar connection between Heimaey and Landeyjahöfn in Iceland. She was delivered from Crist Shipyard, Poland in 2019 for the Owner Vegagerðin, the Icelandic Road Authorities.

The vessel is designed and equipped to meet these conditions by excellent manoeuvrability and shallow water seakeeping.

The vessel is fully electric with a 3 MWh battery pack and rotatable pulling thrusters of 2 x 1700 kW.

# General data about PK

Polarkonsult AS was established in 1974 and is one of the leading independent consultant companies within ship design and naval architecture in Norway. We deliver ship design, inspections and survey for the global market. Engineering and documentation for the energy, oil and gas sector is a growing activity.

​

Our customers’ requirements have top priority and our philosophy is to be a reliable and competent supplier of modern technology for the future.